DAILY ASSESSMENT FORMAT

| Date: | 10-06-2020 | Name: | Anand kumar k |
| --- | --- | --- | --- |
| Course: |  | USN: | 4al16ec002 |
| Topic: | VLSI | Semester & Section: | 8thsem ‘A’ sec |
| Github Repository: | Anand-courses |  |  |

FORENOON SESSION DETAILS

Image of session The working of a MOSFET depends upon the MOS capacitor. The MOS capacitor is the main part of MOSFET. The semiconductor surface at the below oxide layer which is located between source and drain terminals. It can be inverted from p-type to n-type by applying positive or negative gate voltages. When we apply positive gate voltage the holes present under the oxide layer with a repulsive force and holes are pushed downward with the substrate. The depletion region populated by the bound negative charges which are associated with the acceptor atoms. The electrons reach the channel is formed. The positive voltage also attracts electrons from the n+ source and drain regions into the channel. Now, if a voltage is applied between the drain and source, the current flows freely between the source and drain and the gate voltage controls the electrons in the channel. If we apply negative voltage, a hole channel will be formed under the oxide layer. ### P-Channel MOSFET P-Channel MOSFET The drain and source are heavily doped p+ region and the substrate is in n-type. The current flows due to the flow of positively charged holes also known as p-channel MOSFET. When we apply negative gate voltage, the electrons present beneath the oxide layer experience repulsive force and they are pushed downward in to the substrate, the depletion region is populated by the bound positive charges which are associated with the donor atoms. The negative gate voltage also attracts holes from p+ source and drain region into the channel region. ### N-Channel MOSFET N-Channel MOSFET The drain and source are heavily doped n+ region and the substrate is p-type. The current flows due to the flow of negatively charged electrons, also known as n-channel MOSFET. When we apply the positive gate voltage the holes present beneath the oxide layer experience repulsive force and the holes are pushed downwards in to the bound negative charges which are associated with the acceptor atoms. The positive gate voltage also attracts electrons from n+ source and drain region in to the channel thus an electron reach channel is formed.

| Date: | 10-06-2020 | Name: | Anand kumar k |  |
| --- | --- | --- | --- | --- |
| Course: |  | USN: | 4al16ec002 |  |
| Topic: | mysql | Semester & Section: | 8thsem ‘A’ sec |  |
| AFTERNOON SESSION DETAILS |  |  |  |
| Image of session |  |  |  |
| # MySQL: Joins This MySQL tutorial explains how to use MySQL JOINS (inner and outer) with syntax, visual illustrations, and examples. Description MySQL JOINS are used to retrieve data from multiple tables. A MySQL JOIN is performed whenever two or more tables are joined in a SQL statement. There are different types of MySQL joins: - MySQL INNER JOIN (or sometimes called simple join) - MySQL LEFT OUTER JOIN (or sometimes called LEFT JOIN) - MySQL RIGHT OUTER JOIN (or sometimes called RIGHT JOIN) So let's discuss MySQL JOIN syntax, look at visual illustrations of MySQL JOINS, and explore MySQL JOIN examples. INNER JOIN (simple join) Chances are, you've already written a statement that uses a MySQL INNER JOIN. It is the most common type of join. MySQL INNER JOINS return all rows from multiple tables where the join condition is met. ### Syntax The syntax for the INNER JOIN in MySQL is: SELECT columns FROM table1 INNER JOIN table2 ON table1.column = table2.column; ### Visual Illustration In this visual diagram, the MySQL INNER JOIN returns the shaded area: The MySQL INNER JOIN would return the records where table1 and table2 intersect. ### Example Here is an example of a MySQL INNER JOIN: SELECT suppliers.supplier_id, suppliers.supplier_name, orders.order_date FROM suppliers INNER JOIN orders ON suppliers.supplier_id = orders.supplier_id; This MySQL INNER JOIN example would return all rows from the suppliers and orders tables where there is a matching supplier_id value in both the suppliers and orders tables. If we run the MySQL SELECT statement (that contains an INNER JOIN) below: SELECT suppliers.supplier_id, suppliers.supplier_name, orders.order_date FROM suppliers INNER JOIN orders ON suppliers.supplier_id = orders.supplier_id; ### Old Syntax As a final note, it is worth mentioning that the MySQL INNER JOIN example above could be rewritten using the older implicit syntax as follows (but we still recommend using the INNER JOIN keyword syntax): SELECT suppliers.supplier_id, suppliers.supplier_name, orders.order_date FROM suppliers, orders WHERE suppliers.supplier_id = orders.supplier_id; ## LEFT OUTER JOIN Another type of join is called a MySQL LEFT OUTER JOIN. This type of join returns all rows from the LEFT-hand table specified in the ON condition and only those rows from the other table where the joined fields are equal (join condition is met). ### Syntax The syntax for the LEFT OUTER JOIN in MySQL is: SELECT columns FROM table1 LEFT [OUTER] JOIN table2 ON table1.column = table2.column; In some databases, the LEFT OUTER JOIN keywords are replaced with LEFT JOIN. |  |  |  |